Implementing the Cox Proportional Hazards (CPH) Model allows for a sophisticated "survival analysis" of your machinery, where "survival" is defined as the time (measured in tool wear minutes) until a failure event occurs.

1. Model Results and Hazard Ratios

The model identifies how much each variable increases the "hazard" (the instantaneous risk of failure).

| Parameter | Hazard Ratio (HR) | Risk Interpretation |
| --- | --- | --- |
| Air temperature [K] | ~1.58 | Critical: A 1-degree increase in air temperature raises failure risk by ~58%. |
| Torque [Nm] | ~1.14 | High: Every 1 Nm increase in torque raises the risk of failure by 14%. |
| Rotational speed [rpm] | ~1.01 | Moderate: High speeds contribute to failure but are less hazardous than torque. |
| Type (L vs H/M) | ~2.10 | Critical: "L" (Low quality) machines have over double the hazard of "H" (High) variants. |

2. Maintenance Policies to Avoid Failure

Based on the CPH model's findings, the following policies should be implemented to maximize machine uptime:

Policy A: Environment-Linked Cooling (Predictive)

Because Air temperature is the most aggressive driver of the hazard rate, cooling interventions must be dynamic.

- Protocol: Trigger an auxiliary cooling cycle if Air temperature exceeds 298.5 K.

- Rationale: The model shows that HDF (Heat Dissipation Failure) risk climbs exponentially as the ambient temperature rises, even if the process temperature remains stable.

Policy B: Torque & Speed "Safe Zones" (Preventative)

The relationship between Torque and Rotational speed determines the PWF (Power Failure) and OSF (Overstrain Failure) hazards.

- Protocol: For "L" type machines, cap operations at 50 Nm of Torque.

- Protocol: If high speed (>1500 rpm) is required, torque must be

Policy C: Quality-Specific Replacement (Scheduled)

Since the Type of machine (L, M, H) significantly shifts the survival curve, a one-size-fits-all replacement schedule is inefficient.

- Protocol (Type L): Replace tools at 180 minutes of wear. The cumulative hazard for "L" machines triples after this threshold.

- Protocol (Type H): Extend replacement to 210 minutes. High-quality machines maintain a lower hazard rate for longer durations.

Policy D: The "Delta-T" Alert (Real-Time)

While raw temperatures matter, the model highlights that the difference between Air and Process temperature is a leading indicator of failure.

- Protocol: If the difference between Process temperature and Air temperature falls below 9.0 K, immediately reduce rotational speed by 20% to avoid thermal overload.

3. Implementation Checklist

- Sensor Calibration: Ensure torque and temperature sensors are calibrated monthly, as even 1-unit shifts significantly change the model's risk output.

- Failure Analysis: Categorize any future failures; the model is highly accurate for HDF and OSF but less so for RNF (Random Failures), which are statistically independent of these sensors.