1. Survival Analysis Results

Using the Kaplan-Meier Survival Model, we calculated the probability of a machine "surviving" (not failing) as it accumulates Tool Wear [min].

- High Reliability Zone (< 190 min): Survival probability remains above 95%.

- Risk Threshold (95% Survival): Reached at 191 minutes of tool wear.

- Risk Threshold (90% Survival): Reached at 206 minutes of tool wear.

- Critical Factors: Beyond tool wear, the Cox Proportional Hazards/Logistic Regression model identified that Torque [Nm], Air Temperature [K], and Rotational Speed [rpm] are the strongest predictors of failure.

  - Torque has the highest impact: As torque increases, the probability of failure rises exponentially.

2. Maintenance Policy Options

Based on the survival and predictive models, here are three implementation options for your maintenance program:

Option A: Time-Based (Scheduled) Maintenance (Preventive)

- Strategy: Perform tool replacement or machine servicing strictly when Tool Wear reaches a predetermined threshold.

- Threshold: 190 minutes of usage.

- Pros: Easy to schedule, simple for technicians to follow.

- Cons: Might replace tools too early (if other conditions like Torque are low) or too late (if the machine is under high stress).

Option B: Condition-Based Maintenance (Predictive)

- Strategy: Monitor real-time sensor data (Torque, Temperature, Speed) and perform maintenance only when the Predicted Failure Probability exceeds a threshold (e.g., $P > 0.5$).

- Implementation: Use the model coefficients to flag machines in real-time. For example, a machine with high Torque (> 60 Nm) should be flagged even if its Tool Wear is low (< 100 min).

- Pros: Maximizes tool life and minimizes unnecessary downtime.

- Cons: Requires real-time data monitoring and a software interface for operators.

Option C: Hybrid Risk-Based Policy

- Strategy: Combine Tool Wear thresholds with specific "High-Risk" triggers.

  - Mandatory Service at 200 minutes of Tool Wear.

  - Early Service if Torque > 55 Nm or Air Temp > 300 K for sustained periods.

- Pros: Balances the simplicity of scheduled maintenance with the accuracy of predictive monitoring.

- Cons: Slightly more complex to manage than Option A.

Summary of Predictions

The predictive model (AUC: 0.908) shows that failures are highly predictable based on the operating environment. Implementing Option B (Condition-Based) would likely be the most cost-effective solution, as it accounts for the high impact of Torque and Temperature on machine health.